Response to Reviewers for GC-ART-01-2024-000392.R1

Comments in blue, replies in black, and actions in bold. Page and line numbers refer to the main article with highlighted changes.

Reviewer #1

1.0. The authors have addressed all the comments raised by the reviewers and hence I am happy to recommend this manuscript for publication.

We thank the Reviewer for their thorough review and recommendation. We appreciate their acknowledgement of our efforts in addressing the raised comments.

Reviewer #2

2.0. The authors’ have revised the manuscript and responded to my comments. Overall, I am happier with the paper as it is not clearer how the pLCA was carried out. Despite this there are still areas which require further clarification or amendment:

We would like to sincerely thank the Reviewer for the points raised and helpful suggestions for improving our manuscript. We hope that the following responses and actions, address all issues of concern.

2.1. ESI - Section 7- gas and electricity prices. EIA annual energy outlook energy price data is for the US only (global projections on demand and trade). How was US prices adapted for a global context?

As pointed out by the reviewer, the gas and electricity prices utilised in the prospective techno-economic assessment shown in Section 7 of the ESI are primarily based on prices for the US only. We acknowledge that these prices cannot be accurately adapted for a global context due to the lack of data availability and the inherent complexities of using future projections to perform a prospective techno-economic assessment, as mentioned in Section 7 of the ESI. We have modified Section 7 of the ESI to clarify that the natural gas and electricity prices are for the US and only provide an approximation to the global context.

2.2. The analysis is a global one and this should be reflected in the title of the paper and mentioned in the methodology, as otherwise it isn’t until the results section that it’s made clear the study is global.

We would like to inform the Reviewer that the analysis is not only on a global scale. We further enhance the dimensionality by conducting prospective life cycle assessments of all regions as defined by the IMAGE IAM utilised in our study. In the main manuscript, the results shown in Fig. 1, 2, and 3 are on a global scale, whereas the results shown in Fig. 4 are performed on a regional scale. Therefore, we have amended the methods section of the main manuscript to more effectively communicate the scope of our study (page 2, line 1, right column).

2.3. ESI section 2.1- 25 bar hydrogen delivered is chosen but for the green hydrogen route (PEM) only 30 bar delivery is available in premise (I am using v1.8.2.dev).

We thank the Reviewer for highlighting this discrepancy. Fossil and blue hydrogen are delivered at 25 bar, while green hydrogen from PEM-based water electrolysis is delivered at 30 bar. The ESI has been updated to clarify and correct the delivery pressures of hydrogen. It should also be noted that the subsequent green ammonia and methanol production routes take hydrogen at 30 bar as an input, which adheres to our results and discussions.

2.4. Was literature data used to build a carbon capture process or was a premise generated process used? If a premise generated process, which one?

As described in Section 2.2 and Table S1 of the ESI, CO2 is assumed to be captured through a mature technology, specifically monoethanolamine-based chemical absorption. The carbon capture and storage inventories, which involve capturing CO2 through monoethanolamine-based chemical absorption, were extracted from the literature [Volkart et al., Int. J. Greenh. Gas Control, 2013, 13, 91–106]. In response to the Reviewers’ suggestions and feedback in comments 2.6 and 2.7, the main processes used in our study have been listed in Table S1.

2.5. ESI- Section 2.3- how was CCS applied to methanol synthesis from natural gas? There is little to zero CO2 produced in methanol synthesis from natural gas as the carbon contained in the natural gas is contained in the methanol. Ecoinvent data (and transformed premise data) does not list any CO2 emissions (but a small amount of methane).

We agree with the Reviewer that there is little to zero CO2 produced in methanol synthesis from natural gas. However, the heat from natural gas used in the methanol production process generates CO2 emissions. As described in Section 2.2, blue scenarios for ammonia and methanol are defined as CO2 capture from both syngas and the flue gas resulting from the combustion of natural gas in the SMR process. The ESI Section 2.3 has been updated to clarify the definition of the blue scenario for methanol.

2.6. ESI-Table S1. Was a premise process e.g. methanol synthesis and distillation, hydrogen from electrolysis, CO2 from DAC, used to model the green methanol routes? It is not clear from the table.

In accordance with Section 2.3 of the ESI, the inventories for the green methanol scenarios were sourced from Hank et al., with modifications. Specifically, the inventories for ‘methanol synthesis and distillation, hydrogen from electrolysis, and CO2 from DAC’ were utilised from this source in premise. However, the inventories for CO2 from DAC and hydrogen from PEM electrolysis, as detailed in Table S1 of the ESI, were substituted for the existing CO2 and hydrogen inventories. We acknowledge the Reviewer’s suggestion regarding the enhancement of inventory source transparency in Table S1. Consequently, we have revised Table S1 in the ESI to provide additional details on the premise and Ecoinvent processes utilised.

2.7. ESI-Table S1- can the premise and ecoinvent processes used (not all but the main ones) be listed?

We have modified Table S1 in the ESI to include more details on the main inventories utilised from premise and Ecoinvent.

2.8. Negative emissions from DAC CO2 - I appreciate the clarifications made in the paper about the cradle to gate system boundary but this result is still highly dependent to the assumption that CO2 from DAC can be considered negative when it’s not being stored. The authors would be better off clarifying this as a limitation of the study and that this particular result is highly subjective to this assumption. There is little consensus on whether CO2 from DAC used in fuel or chemical production would be considered negative and what negative emissions truly means. Based on a report by the USA National Academies of Sciences: 'To be considered a NET, direct air capture systems must sequester the captured CO2 on a timescale that positively impacts climate change' (Direct Air Capture - Negative Emissions Technologies and Reliable Sequestration - NCBI Bookshelf (nih.gov)). In a recent paper which looked at carbon storage (of industrial CO2) in crops, a net carbon storage quantity was determined (https://www.sciencedirect.com/science/article/pii/S0959652623038088#sec2).

As described in the main manuscript, the green methanol routes utilise CO2 from DAC. We agree with the Reviewer that CO2 from DAC technology will only result in negative emissions if captured from the air and sequestered. However, on a cradle-to-gate basis, for green methanol production, the CO2 is indeed sequestered in the form of methanol. This sequestration effectively results in negative emissions being embodied within green methanol. In other words, the CO2 that would otherwise contribute to atmospheric GHG levels is being captured and stored in a stable form. The main manuscript has been amended to further clarify this discussion (page 2, line 39, right column).

2.9. This paper was recently published: https://www.sciencedirect.com/science/article/pii/S2405844024035783?via%3Dihub. It may be useful to compare your results to it.

We thank the Reviewer for bringing this research article to our attention. Even though the scopes of the two papers are different, the results discussed for the prospective life cycle assessment of ammonia production technologies are quite similar. The mentioned work uses the IEA Ammonia Technology Roadmap (8 technologies) in combination with the future electricity mixes from the IMAGE IAM for 26 regions worldwide. This work finds that the impact of the SMR-based fossil ammonia shows little to no improvement in climate change impacts under the 3.5 °C scenario. This finding is similar to our results presented in Fig. 1 of the main manuscript. A comparison for the electrolysis-based ammonia production pathways is not possible due to the use of grid electricity (‘yellow hydrogen’) in the mentioned work, whereas, in our work, we utilise green hydrogen (solar or wind-based). In response to the Reviewer’s suggestion, we have now included a comparison between the results in the main manuscript (page 2, line 43, right column).