Response to Reviewers for GC-ART-01-2024-000392.R1

Comments in blue, replies in black, and actions in bold. Page and line numbers refer to the main article with highlighted changes.

Reviewer #1

1.0. The authors have addressed all the comments raised by the reviewers and hence I am happy to recommend this manuscript for publication.

We thank the Reviewer for their recommendation. We appreciate their acknowledgement of our efforts in addressing the raised comments.

Reviewer #2

2.0. The authors’ have revised the manuscript and responded to my comments. Overall, I am happier with the paper as it is not clearer how the pLCA was carried out. Despite this there are still areas which require further clarification or amendment:

We would like to sincerely thank the Reviewer for the points raised and helpful suggestions for improving our manuscript. We hope that the following responses and actions address all issues of concern.

2.1. ESI - Section 7- gas and electricity prices. EIA annual energy outlook energy price data is for the US only (global projections on demand and trade). How was US prices adapted for a global context?

As pointed out by the Reviewer, the gas and electricity prices utilised in the prospective techno-economic assessment shown in Section 7 of the ESI are primarily based on prices for the US only due to the lack of data availability for other regions and the inherent complexities of using future projections to perform a prospective techno-economic assessment, as mentioned in the same section. However, we believe that the incorporation of sensitivity analysis assuming a 50% deviation as shown in Fig. S7 of the ESI highlights that trends observed would remain the same in the case of using regionalised datasets. We have modified Section 7 of the ESI to clarify that US prices were used for natural gas and electricity, thus providing an approximation to the global context.

2.2. The analysis is a global one and this should be reflected in the title of the paper and mentioned in the methodology, as otherwise it isn’t until the results section that it’s made clear the study is global.

We would like to underscore that the analysis extends beyond the global scale to consider regional features (i.e., utilising global as well as region specific inventories). We enhance granularity by conducting prospective life cycle assessments of all regions as defined by the IMAGE IAM utilised in our study. In the main manuscript, the results shown in Fig. 1, 2, and 3 are on a global scale, whereas the results shown in Fig. 4 are performed on a regional scale. We also feel the original title reflects the generality of the analysis. Therefore, we have amended the methods section of the main manuscript to further clarify the scope of our study (page 2, line 1, right column).

2.3. ESI section 2.1- 25 bar hydrogen delivered is chosen but for the green hydrogen route (PEM) only 30 bar delivery is available in premise (I am using v1.8.2.dev).

We thank the Reviewer for highlighting this discrepancy. Fossil and blue hydrogen are delivered at 25 bar, while green hydrogen from PEM-based water electrolysis is delivered at 30 bar. The ESI has been updated to clarify and correct the delivery pressures of hydrogen. It should also be noted that the subsequent green ammonia and methanol production routes take hydrogen at 30 bar as an input, which adheres to our results and discussions. Therefore, the key conclusions of our study remain unchanged despite this modification.

2.4. Was literature data used to build a carbon capture process or was a premise generated process used? If a premise generated process, which one?

CO2 is assumed to be captured through a mature technology, specifically monoethanolamine-based chemical absorption (Section 2.2 and Table S1 of the ESI). The carbon capture and storage inventories were modelled based on literature [Volkart et al., Int. J. Greenh. Gas Control, 2013, 13, 91–106] as already cited in Section 2.2 and Table S1 of the ESI, and were not generated by premise. In response to the Reviewers’ suggestions and feedback provided in comments 2.6 and 2.7, the main processes used in our study have been listed in Table S1.

2.5. ESI- Section 2.3- how was CCS applied to methanol synthesis from natural gas? There is little to zero CO2 produced in methanol synthesis from natural gas as the carbon contained in the natural gas is contained in the methanol. Ecoinvent data (and transformed premise data) does not list any CO2 emissions (but a small amount of methane).

We agree with the Reviewer that there is little to zero CO2 produced in the chemical reaction producing methanol from natural gas. However, the heat in methanol production (for SMR and the reboiler of the methanol-water column) commonly originates from natural gas, resulting in net CO2 emissions. Section 2.2 describes how blue scenarios for ammonia are defined as CO2 capture from both syngas and flue gas resulting from the combustion of natural gas in the SMR process. On the other hand, blue scenarios for methanol are defined as CO2 capture from flue gas only. Section 2.3 of the ESI has been updated to clarify the definition of the blue scenario for methanol.

2.6. ESI-Table S1. Was a premise process e.g. methanol synthesis and distillation, hydrogen from electrolysis, CO2 from DAC, used to model the green methanol routes? It is not clear from the table.

The inventories for the green methanol scenarios were sourced from Hank et al., with modifications (Section 2.3 of the ESI). Specifically, the inventories for ‘methanol synthesis and distillation, hydrogen from electrolysis, and CO2 from DAC’ from premise were utilised. However, in the methanol inventories from premise, the CO2 from DAC and hydrogen from PEM electrolysis were replaced with alternative CO2 and hydrogen inventories as detailed in Table S1 of the ESI. We acknowledge the Reviewer’s suggestion regarding further strengthening the transparency of inventory sources in Table S1. Consequently, we have revised Table S1 in the ESI to provide additional details on the premise and Ecoinvent processes utilised.

2.7. ESI-Table S1- can the premise and ecoinvent processes used (not all but the main ones) be listed?

We have modified Table S1 in the ESI to include more details on the main inventories utilised from premise and Ecoinvent.

2.8. Negative emissions from DAC CO2 - I appreciate the clarifications made in the paper about the cradle to gate system boundary but this result is still highly dependent to the assumption that CO2 from DAC can be considered negative when it’s not being stored. The authors would be better off clarifying this as a limitation of the study and that this particular result is highly subjective to this assumption. There is little consensus on whether CO2 from DAC used in fuel or chemical production would be considered negative and what negative emissions truly means. Based on a report by the USA National Academies of Sciences: 'To be considered a NET, direct air capture systems must sequester the captured CO2 on a timescale that positively impacts climate change' (Direct Air Capture - Negative Emissions Technologies and Reliable Sequestration - NCBI Bookshelf (nih.gov)). In a recent paper which looked at carbon storage (of industrial CO2) in crops, a net carbon storage quantity was determined (https://www.sciencedirect.com/science/article/pii/S0959652623038088#sec2).

We agree with the Reviewer that CO2 from DAC technology will only result in negative emissions if captured from the air and sequestered. However, on a cradle-to-gate basis, the CO2 is contained in the methanol. Nonetheless, we acknowledge that on a cradle-to-grave basis, this CO2 might return to the atmosphere. The main manuscript has been amended to further clarify this discussion (page 2, line 38, right column). Additionally, the use of “net captured CO2” or “captured CO2” while describing the negative climate change impacts for methanol has been modified to “net negative emissions” or “negative emissions” on a cradle-to-gate basis throughout the manuscript to enhance clarity. We also thank the Reviewer for sharing the research article on the use of industrial CO2 in crops for achieving a circular economy. This work is now cited in the discussion section of the main manuscript (page 5, line 12, right column).

2.9. This paper was recently published: https://www.sciencedirect.com/science/article/pii/S2405844024035783?via%3Dihub. It may be useful to compare your results to it.

We thank the Reviewer for bringing this research article to our attention. The mentioned work provides a comprehensive outlook on different ammonia production technologies and the challenges of decarbonising the ammonia industry based on the IEA Ammonia Technology Roadmap by performing a prospective LCA on global ammonia production. This work finds that the impact of the SMR-based fossil ammonia shows little to no improvement in climate change impacts under the 3.5 °C scenario. This finding is similar to our results presented in Fig. 1 of the main manuscript. A comparison of the electrolysis-based ammonia production pathways is not feasible due to the use of grid electricity (‘yellow hydrogen’) in the mentioned work, whereas our study employs green hydrogen derived from solar or wind sources. This distinction represents an enhancement in our approach, performing a more comprehensive evaluation of green ammonia production pathways. Furthermore, in addition to ammonia, we perform prospective LCAs for hydrogen and methanol production pathways. In response to the Reviewer’s suggestion, we have now included a comparison between the results in the main manuscript (page 2, line 42, right column) and also cite this work in our introduction (page 2, line 17, left column).